Worksheet 2 Questions and Answers

Exploring the ARM Cortex M3

Question 1: Which devices have ARM chips in them, and who makes the chips?

I found that many everyday devices use ARM-based processors because ARM designs are efficient, low-power and suitable for embedded and mobile systems. Examples include mobile phones, tablets, smart TVs, home routers, Raspberry Pi boards and the Olimex STM32-P107 board used in this worksheet.

In mobile phones, ARM-based chips are commonly made by companies such as Qualcomm, Samsung, Apple and MediaTek. In routers and smart TVs, ARM-based SoCs are commonly made by Broadcom, MediaTek, Realtek and Qualcomm Atheros. In the Olimex STM32-P107 board, the processor is the STMicroelectronics STM32F107VCT6, which is based on the ARM Cortex-M3 core.

This is relevant to the worksheet because the Olimex board is a microcontroller system, and the Cortex-M3 is designed for small embedded systems with peripherals, low power use and efficient code execution.

Question 2: What do the GCC command flags mean?

I checked the compiler options used in the Makefile and identified the purpose of each important flag. These flags are needed because I am cross-compiling on a Linux PC for an ARM Cortex-M3 target rather than compiling for the host computer.

-mcpu=cortex-m3 tells GCC to generate code for the ARM Cortex-M3 CPU.

-mthumb tells GCC to generate Thumb or Thumb-2 instruction code, which is used by the Cortex-M3.

-O1 enables optimization level 1. It improves the code while still keeping debugging reasonably clear.

-g adds debugging information so GDB can show source lines, variables and symbols.

-T stm32f100.ld tells the linker to use the custom linker script for the STM32 board memory layout.

-I adds include directories so the compiler can find header files such as stm32f10x.h.

-DSTM32F10X_CL defines the target device family as the STM32 connectivity line.

-DUSE_STDPERIPH_DRIVER enables the ST Standard Peripheral Library.

-DUSE_FULL_ASSERT enables full assertion checking in the ST library.

The two most important target-specific flags are -mcpu=cortex-m3 and -mthumb because they make sure the output matches the ARM Cortex-M3 architecture on the Olimex board.

Question 3: How can I stop the compilation process at different stages, and what are those stages?

I used GCC options to stop the compilation process at different stages. This helped me understand the build pipeline from C source code to the final executable.

-E stops after preprocessing. It expands include files and macros and produces preprocessed C output, usually saved as a .i file.

-S stops after compilation. It converts the C source into assembly language, usually saved as a .s file.

-c stops after assembling. It produces an object file, usually saved as a .o file, but does not link the final executable.

Running GCC without these stop options allows the build to continue through linking and create the final executable, which is demo.elf in this worksheet.

The commands I used were:

arm-none-eabi-gcc -E -mcpu=cortex-m3 -mthumb main.c -o main.i
arm-none-eabi-gcc -S -mcpu=cortex-m3 -mthumb main.c -o main.s
arm-none-eabi-gcc -c -mcpu=cortex-m3 -mthumb main.c -o main_test.o

The files main.i, main.s and main_test.o show the different outputs created at each stage.

Question 4: How do I build a version of the binary for a debugger?

I build a debugger-friendly version by compiling with the -g flag. This includes debugging symbols in the ELF file so GDB can understand function names, variable names and source line numbers.

The Makefile already uses:

CFLAGS = -O1 -g

This is already suitable for debugging. For even clearer debugging, I can temporarily use:

CFLAGS = -O0 -g

The -O0 option disables optimization, so GDB follows the source code more directly. After demonstrating this, I restored the Makefile to CFLAGS = -O1 -g because that was the original worksheet configuration and it still supports debugging.

Question 5: How many ARM CPUs are supported by the installed toolchain?

I attempted to list supported ARM CPUs using:

arm-none-eabi-gcc -mcpu=help

In my toolchain version, GCC reported that help was not a valid CPU name, but it still printed the list of valid CPU arguments in the error message. This list included many ARM cores such as arm7tdmi, arm9, cortex-a5, cortex-a7, cortex-a8, cortex-a9, cortex-r4, cortex-r5, cortex-m0, cortex-m1, cortex-m3, cortex-m4, cortex-m7, cortex-m23 and cortex-m33.

This confirmed that cortex-m3 is supported by the installed ARM toolchain, which is the target CPU required for the Olimex STM32-P107 board.

I also used:

arm-none-eabi-gcc --target-help | grep -A40 mcpu

This showed that -mcpu specifies the target CPU name and -mthumb generates Thumb-state code.

Question 6: What do the clean and debug labels in the Makefile do?

I inspected the Makefile labels and tested them. The clean label removes generated build files, such as object files, dependency files and the final demo.elf executable. This lets me rebuild the project from a clean state.

The debug label is intended to start GDB using the generated ELF file. In my setup, the Makefile originally tried to run arm-none-eabi-gdb, but that command was not installed. I changed the debug rule to use gdb-multiarch instead, which is available on my VM and works with the ARM ELF file.

The debug rule became:

debug: $(ELF)
	gdb-multiarch $(ELF)

After this change, make debug opened demo.elf in GDB correctly.

Question 7: How did I modify main.c to include another variable k that is also incremented?

I modified main.c by adding a new global variable k and incrementing it inside inc() along with i. This allowed me to debug two changing variables instead of one.

The modified program was:

int i = 0;
int k = 0;
int off = 5;

void inc(void)
{
    i += off;
    k += 1;
}

int main(void)
{
    while (1) {
        inc();
    }
}

After editing the file, I rebuilt the program using make clean and make, then reloaded demo.elf onto the board through OpenOCD.

Question 8: How did I set up a watchpoint on i or k?

In GDB, I used hardware watchpoints to stop execution when a variable changed. I first used:

watch i
continue

GDB created a hardware watchpoint on i. When the program continued, it stopped with SIGTRAP when i changed inside inc(). I then tested the same idea with:

watch k
continue

This created another hardware watchpoint and stopped the program when k changed.

Question 9: How did I print register values in GDB?

I printed CPU register values using GDB commands. For example, I used:

print /x $sp
print /x $pc
print /x $lr
info registers

The /x option prints values in hexadecimal. The stack pointer is $sp, the program counter is $pc, and the link register is $lr.

These registers are important on ARM Cortex-M3 because the stack pointer tracks the current stack location, the program counter shows the address of the current instruction, and the link register stores the return address after function calls.

Question 10: How did I use a breakpoint command that prints i and continues before inc() runs?

I used a GDB breakpoint command list to automatically print the value of i whenever execution reached inc(), then continue running without manually typing continue each time.

The commands were:

delete breakpoints
break inc
commands
silent
print i
continue
end
continue

This means that each time inc() is reached, GDB does not print the normal breakpoint message, prints i, and then continues automatically. I stopped the repeated output using Ctrl+C when enough evidence had been collected.